Booth		Ge0/0/0 - 0/0/1 	Trunk Uplink to Catwalk ?

Ge0/0/2 - 0/0/7 	Reserved ?

Ge0/0/8 - 0/0/17	Trunk AP’s

Ge0/0/18 - 0/0/47 	Disabled ?

Catwalk	Ge0/0/0		Trunk Link to EX Router

Ge0/0/1		Trunk Downlink to Ballroom F

Ge0/0/2 		Trunk Downlink to Ballroom G

Ge0/0/3		Trunk Downlink to Ballroom H

Ge0/0/4 - 0/0/29	Trunk Downlinks to Booths

Ge0/0/30 - 0/0/47	Disabled

cfAV		Ge0/0/0		Trunk Uplink

Ge0/0/1		Trunk Downlink

Ge0/0/2 - 0/0/7	Trunk AP’s

Ge0/0/8 - 0/0/11 	Digital Signage

Ge0/0/12 - 0/0/47 	AV VLAN

cfCTF		Ge0/0/0 - 0/0/47 	CTF VLAN

cfIDF		Ge0/0/0 - 0/0/1	Trunk Link to CF Router

Ge0/0/2 - 0/0/17	Trunk

Ge0/0/18 - 0/0/21 	CTF VLAN

Ge0/0/22 - 0/0/47	Disabled

cfRoom	Ge0/0/0		Trunk Uplink

Ge0/0/1		Trunk Downlink

Ge0/0/2 - 0/0/7	Reserved

Ge0/0/8 - 0/0/17	Trunk AP’s

Ge0/0/18 - 0/0/21	Digital Signage VLAN

Ge0/0/22 - 0/0/27	AV VLAN

Ge0/0/28 - 0/0/29	Speaker VLAN

Ge0/0/30		N6S Station

Ge0/0/31		HAM Link to N6S to Expo

exIDF		Ge0/0/0		Trunk Link to EX Router

Ge0/0/1		Trunk Downlink to Ballroom F

Ge0/0/2		Trunk Downlink to Ballroom G

Ge0/0/3		Trunk Downlink to Ballroom H

Ge0/0/4 - 0/0/7 	Trunk Unassigned

Ge0/0/8 - 0/0/17	Trunk AP’s

Ge0/0/18 - 0/0/21	Digital Signage VLAN

Ge0/1/0		Fiber Uplink to exMDF switch

exRoom	Ge0/0/0		Trunk Uplink

Ge0/0/1		Trunk Uplink

Ge0/0/2 - 0/0/7	Reserved

Ge0/0/8 - 0/0/17	Trunk AP’s

Ge0/0/18 - 0/0/21	Digital Signage VLAN

Ge0/0/22 - 0/0/27	AV VLAN

Ge0/0/28 - 0/0/29 	Speaker VLAN

Ge0/0/30 - 0/0/47	Disabled

Registration	Ge0/0/0 - 0/0/1	Trunk Uplink

Ge0/0/2 - 0/0/7	Reserved

Ge0/0/8 - 0/0/15	Trunk AP’s

Ge0/0/16 - 0/0/17	Digital Signage VLAN

Ge0/0/18 - 0/0/19	Reserved

Ge0/0/20 - 0/0/39 	Registration VLAN

Ge0/0/40 - 0/0/46	Reserved

Ge0/0/47		Expo Infrastructure VLAN 1